CS-350-11992-M01 Emerging Sys Arch & Tech 2025 C

CS 350

Module 7

7-1 Final Project: Thermostat Lab

CS 350 Final Project

Title: CS 350 Final Project – Thermostat Prototype
Author: Marissa Lanza
Date: June 23, 2025

Introduction

This prototype implements the low-level logic of a three-state (OFF/HEATING/COOLING) thermostat entirely in Python. Because I did not have a Raspberry Pi or the AHT20 sensor and LED/button hardware on hand, I developed import stubs for all GPIO, I²C, and UART libraries. I confirmed this approach with my professor, who indicated that the primary deliverables are:

- The Python code (main.py),

- The state-machine diagram exported as PDF (state_machine.pdf), and

- This written reflection (.docx).

Functionality & Stub Strategy

- State Machine cycles through OFF → HEATING → COOLING → OFF via a “mode” button stub.

- Set-Point Adjustment with “Up”/“Down” button stubs.

- Temperature Reading returns a placeholder drift (72–73 °F); on actual hardware, replace with real AHT20 I²C code.

- LED Output simulated by no-op PWMLED stubs. Heating pulses red, cooling pulses blue, solid when at set-point.

- Display uses print() to simulate the LCD (timestamp, mode, temperatures).

- UART stub writes comma-delimited status strings to a dummy serial port, simulating server communication.

All hardware-dependent imports are wrapped in try/except ImportError so the logic can be fully tested on Windows.

State-Machine Diagram

(Include the one-page PDF you exported from draw.io here.)
The diagram shows transitions on mode‐button presses and how each state maps to the LED and temperature-comparison logic.

Code Quality & Best Practices

- Modular design: separate methods for sensing, actuation, display, and communication.

- Descriptive naming: e.g. toggle_mode(), update_leds().

- In-line comments clarify sensor stubs and timing choices.

- Testability: shortened loop delay (time.sleep(5)) for rapid local testing.

Next Steps & Hardware Integration

- When hardware arrives: remove stubs, install gpiozero, smbus2, and pyserial, hook up buttons/LEDs/AHT20 per lab guide.

- LCD driver: swap print() for an actual LCD‐driver library.

- Cloud connectivity: extend the UART stub to real Wi-Fi (e.g. MQTT).

- Unit tests: add pytest or unittest coverage for state transitions.

My PDF of thermostat lab: